OS MEDIA

# Pro+Grip Screws USA

Full Product Page Content — All Categories

This document contains SEO / AEO product page content for every SKU in the Pro+Grip USA product line, organized to match the site's product category structure. Each product includes a suggested URL slug, SEO title tag, meta description, product overview, full spec table, feature bullets, and an FAQ block written for answer engine visibility.

Site Category Structure

Sharp Point - Fine Thread  (14 SKUs)

•  Drywall Screws  (9 SKUs)

•  Framing Screws  (2 SKUs)

•  Collated Screws  (2 SKUs)F

•  Laminated  (1 SKUs)

Sharp Point - Coarse Thread  (12 SKUs)

•  Drywall Screws  (10 SKUs)

•  Collated Screws  (2 SKUs)

Self Drilling  (15 SKUs)

•  Drywall Screws  (8 SKUs)

•  Framing Screws  (5 SKUs)

•  Collated Screws  (2 SKUs)

Concrete and Cement Board Screws  (10 SKUs)

•  Concrete and Cement Board Screws  (10 SKUs)

Total: 51 SKUs

# Sharp Point - Fine Thread

## Sharp Point - Fine Thread  /  Drywall Screws

# Pro+Grip #6 x 1" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-fine-thread-drywall-screw-drp6100

SEO Title Tag: Pro+Grip #6 x 1" Fine Thread Drywall Screw | Pro+Grip DRP6100

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6100 is a #6 x 1" fine thread screw sized for 20-25 gauge metal framing.

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6100 is a #6 x 1" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 15 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP6100 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | 13.7 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.18-2.33 | Major Diameter (mm) | G2: 3.40-3.55Standard: 3.45-3.58 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 15 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP6100 drywall screw used for?

The Pro+Grip DRP6100 is a #6 x 1" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (15 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP6100 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP6100 screws come in a box?

Each box of DRP6100 contains 10000 screws and weighs approximately 13.7 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #6 x 1-1/8" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-8-fine-thread-drywall-screw-drp6118

SEO Title Tag: Pro+Grip #6 x 1-1/8" Fine Thread Drywall Screw | Pro+Grip DRP6118

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6118 is a #6 x 1-1/8" fine thread screw sized for 20-25 gauge metal framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6118 is a #6 x 1-1/8" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 15 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP6118 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/8" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | 14.7 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.18-2.33 | Major Diameter (mm) | G2: 3.40-3.55Standard: 3.45-3.58 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 15 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP6118 drywall screw used for?

The Pro+Grip DRP6118 is a #6 x 1-1/8" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (15 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP6118 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP6118 screws come in a box?

Each box of DRP6118 contains 10000 screws and weighs approximately 14.7 lbs, with box dimensions of 28*26*15.5.

# Pro+Grip #6 x 1-1/4" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-4-fine-thread-drywall-screw-drp6114

SEO Title Tag: Pro+Grip #6 x 1-1/4" Fine Thread Drywall Screw | Pro+Grip DRP6114

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6114 is a #6 x 1-1/4" fine thread screw sized for 20-25 gauge metal framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6114 is a #6 x 1-1/4" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 15 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP6114 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/4" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 8000 | Weight per Box (lbs) | 13.5 |
| Box Dimensions | 25*26*15.5 | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.18-2.33 | Major Diameter (mm) | G2: 3.40-3.55Standard: 3.45-3.58 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 15 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP6114 drywall screw used for?

The Pro+Grip DRP6114 is a #6 x 1-1/4" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (15 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP6114 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP6114 screws come in a box?

Each box of DRP6114 contains 8000 screws and weighs approximately 13.5 lbs, with box dimensions of 25*26*15.5.

# Pro+Grip #6 x 1-5/8" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-5-8-fine-thread-drywall-screw-drp6158

SEO Title Tag: Pro+Grip #6 x 1-5/8" Fine Thread Drywall Screw | Pro+Grip DRP6158

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6158 is a #6 x 1-5/8" fine thread screw sized for 20-25 gauge metal framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6158 is a #6 x 1-5/8" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 15 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP6158 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-5/8" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 5000 | Weight per Box (lbs) | 10.25 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.18-2.33 | Major Diameter (mm) | G2: 3.40-3.55Standard: 3.45-3.58 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 15 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP6158 drywall screw used for?

The Pro+Grip DRP6158 is a #6 x 1-5/8" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (15 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP6158 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP6158 screws come in a box?

Each box of DRP6158 contains 5000 screws and weighs approximately 10.25 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #6 x 2" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-2-fine-thread-drywall-screw-drp6200

SEO Title Tag: Pro+Grip #6 x 2" Fine Thread Drywall Screw | Pro+Grip DRP6200

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6200 is a #6 x 2" fine thread screw sized for 20-25 gauge metal framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP6200 is a #6 x 2" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 15 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP6200 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 2" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 3500 | Weight per Box (lbs) | 8.87 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.18-2.33 | Major Diameter (mm) | G2: 3.40-3.55Standard: 3.45-3.58 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 15 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP6200 drywall screw used for?

The Pro+Grip DRP6200 is a #6 x 2" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (15 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP6200 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP6200 screws come in a box?

Each box of DRP6200 contains 3500 screws and weighs approximately 8.87 lbs, with box dimensions of 28*26*15.5.

# Pro+Grip #8 x 2-1/2" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-8-x-2-1-2-fine-thread-drywall-screw-drp8212

SEO Title Tag: Pro+Grip #8 x 2-1/2" Fine Thread Drywall Screw | Pro+Grip DRP8212

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP8212 is a #8 x 2-1/2" fine thread screw sized for 20-25 gauge metal framing.

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP8212 is a #8 x 2-1/2" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 15 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP8212 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #8 |
| Length | 2-1/2" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 2500 | Weight per Box (lbs) | 10.81 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.80-3.00 | Major Diameter (mm) | 4.00-4.20 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #8 shank with 15 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP8212 drywall screw used for?

The Pro+Grip DRP8212 is a #8 x 2-1/2" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (15 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP8212 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP8212 screws come in a box?

Each box of DRP8212 contains 2500 screws and weighs approximately 10.81 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip #8 x 3" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-8-x-3-fine-thread-drywall-screw-drp8300

SEO Title Tag: Pro+Grip #8 x 3" Fine Thread Drywall Screw | Pro+Grip DRP8300

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP8300 is a #8 x 3" fine thread screw sized for 20-25 gauge metal framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP8300 is a #8 x 3" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 15 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP8300 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #8 |
| Length | 3" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 2000 | Weight per Box (lbs) | 10.21 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.80-3.00 | Major Diameter (mm) | 4.00-4.20 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #8 shank with 15 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP8300 drywall screw used for?

The Pro+Grip DRP8300 is a #8 x 3" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (15 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP8300 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP8300 screws come in a box?

Each box of DRP8300 contains 2000 screws and weighs approximately 10.21 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip #10 x 3-1/2" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-10-x-3-1-2-fine-thread-drywall-screw-drp10312

SEO Title Tag: Pro+Grip #10 x 3-1/2" Fine Thread Drywall Screw | Pro+Grip DRP10312

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP10312 is a #10 x 3-1/2" fine thread screw sized for 20-25 gauge metal framing.

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP10312 is a #10 x 3-1/2" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 12 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP10312 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #10 |
| Length | 3-1/2" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 1000 | Weight per Box (lbs) | 8.31 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 12 |
| Nominal Head Diameter (mm) | 8.50-8.90 | Recess Depth (mm) | 2.87-3.30 |
| Minor Diameter (mm) | 3.20-3.40 | Major Diameter (mm) | 4.75-4.95 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #10 shank with 12 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP10312 drywall screw used for?

The Pro+Grip DRP10312 is a #10 x 3-1/2" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (12 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP10312 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP10312 screws come in a box?

Each box of DRP10312 contains 1000 screws and weighs approximately 8.31 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip #10 x 6" Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-10-x-6-fine-thread-drywall-screw-drp10600

SEO Title Tag: Pro+Grip #10 x 6" Fine Thread Drywall Screw | Pro+Grip DRP10600

Meta Description: Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP10600 is a #10 x 6" fine thread screw sized for 20-25 gauge metal framing.

## Product Overview

Built for hanging drywall to light-gauge metal studs, the Pro+Grip DRP10600 is a #10 x 6" fine thread screw sized for 20-25 gauge metal framing. Its sharp point and 12 TPI thread deliver clean penetration without stripping the metal track.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP10600 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #10 |
| Length | 6" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 1000 | Weight per Box (lbs) | 13.81 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 12 |
| Nominal Head Diameter (mm) | 8.50-8.90 | Recess Depth (mm) | 2.87-3.30 |
| Minor Diameter (mm) | 3.20-3.40 | Major Diameter (mm) | 4.75-4.95 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #10 shank with 12 TPI fine thread built for clean, strip-free installs into 20-25 gauge metal framing

- Sharp point starts fast without a pilot hole on standard-gauge track and studs

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRP10600 drywall screw used for?

The Pro+Grip DRP10600 is a #10 x 6" fine thread drywall screw designed for fastening drywall to light-gauge metal studs (20-25 gauge). Its sharp point and fine thread pitch (12 TPI) deliver clean penetration without stripping the metal track.

Can this screw be used on wood framing?

Fine thread screws like the DRP10600 are engineered for metal framing. For wood studs, use a coarse thread Pro+Grip drywall screw, which is built to grip wood fiber rather than sheet metal.

How many DRP10600 screws come in a box?

Each box of DRP10600 contains 1000 screws and weighs approximately 13.81 lbs, with box dimensions of 28*26*16.5.

## Sharp Point - Fine Thread  /  Framing Screws

# Pro+Grip #7 x 7/16" Sharp Point Framing Screw

Suggested URL Slug: /products/progrip-7-x-7-16-sharp-point-framing-screw-drwp7716

SEO Title Tag: Pro+Grip #7 x 7/16" Sharp Point Framing Screw | Pro+Grip DRWP7716

Meta Description: The Pro+Grip DRWP7716 is a #7 x 7/16" sharp point framing screw built for fastening metal framing components together, such as track to stud connections, in...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip DRWP7716 is a #7 x 7/16" sharp point framing screw built for fastening metal framing components together, such as track to stud connections, in light-gauge steel framing up to 20-25 gauge.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRWP7716 | Application | Metal to Metal |
| --- | --- | --- | --- |
| Screw Type | Framing Screw | Gauge / Diameter | #7 |
| Length | 7/16" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Pan Framer Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 15000 | Weight per Box (lbs) | 18.2 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 7.40-8.10 | Recess Depth (mm) | 2.49-2.87 |
| Minor Diameter (mm) | 2.38-2.58 | Major Diameter (mm) | 3.65-3.85 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #7 shank with 20 TPI fine thread built for metal-to-metal connections

- Sharp point starts fast in 20-25 gauge steel without a pilot hole

- Pan Framer Head for a low-profile, secure connection between framing members

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the DRWP7716 framing screw used for?

The Pro+Grip DRWP7716 is used to fasten metal framing components together, such as connecting track to stud, in light-gauge steel construction up to 20-25 gauge.

Can I use a sharp point framing screw on heavier gauge steel?

Sharp point screws like the DRWP7716 are built for lighter gauge connections. For heavier steel, use a Pro+Grip self-drilling framing screw, which is built to drill its own pilot hole.

How many DRWP7716 screws come in a box?

Each box of DRWP7716 contains 15000 screws and weighs approximately 18.2 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #7 x 9/16" Sharp Point Framing Screw

Suggested URL Slug: /products/progrip-7-x-9-16-sharp-point-framing-screw-drwp7916

SEO Title Tag: Pro+Grip #7 x 9/16" Sharp Point Framing Screw | Pro+Grip DRWP7916

Meta Description: The Pro+Grip DRWP7916 is a #7 x 9/16" sharp point framing screw built for fastening metal framing components together, such as track to stud connections, in...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip DRWP7916 is a #7 x 9/16" sharp point framing screw built for fastening metal framing components together, such as track to stud connections, in light-gauge steel framing up to 20-25 gauge.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRWP7916 | Application | Metal to Metal |
| --- | --- | --- | --- |
| Screw Type | Framing Screw | Gauge / Diameter | #7 |
| Length | 9/16" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Wafer Head - Modified Truss |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | 18.1 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 11.10-11.60 | Recess Depth (mm) | 2.24-2.74 |
| Minor Diameter (mm) | 2.38-2.58 | Major Diameter (mm) | 3.65-3.85 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #7 shank with 20 TPI fine thread built for metal-to-metal connections

- Sharp point starts fast in 20-25 gauge steel without a pilot hole

- Wafer Head - Modified Truss for a low-profile, secure connection between framing members

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the DRWP7916 framing screw used for?

The Pro+Grip DRWP7916 is used to fasten metal framing components together, such as connecting track to stud, in light-gauge steel construction up to 20-25 gauge.

Can I use a sharp point framing screw on heavier gauge steel?

Sharp point screws like the DRWP7916 are built for lighter gauge connections. For heavier steel, use a Pro+Grip self-drilling framing screw, which is built to drill its own pilot hole.

How many DRWP7916 screws come in a box?

Each box of DRWP7916 contains 10000 screws and weighs approximately 18.1 lbs, with box dimensions of 28*26*13.5.

## Sharp Point - Fine Thread  /  Collated Screws

# Pro+Grip #6 x 1-1/4" Collated Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-4-collated-fine-thread-drywall-screw-drp6114s

SEO Title Tag: Pro+Grip #6 x 1-1/4" Collated Fine Thread Drywall Screw | Pro+Grip DRP6114S

Meta Description: The Pro+Grip DRP6114S takes our #6 x 1-1/4" fine thread drywall screw and loads it into a collated strip, so installers running autofeed screw guns can...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip DRP6114S takes our #6 x 1-1/4" fine thread drywall screw and loads it into a collated strip, so installers running autofeed screw guns can drive fasteners faster with less fatigue on large drywall-to-metal jobs. Same 15 TPI thread and sharp point as our standard fine thread line, just packaged for speed.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP6114S | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Collated Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/4" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.18-2.33 | Major Diameter (mm) | G2: 3.40-3.55Standard: 3.45-3.58 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- Strip-collated for instant loading into autofeed screw gun attachments

- 15 TPI fine thread engineered for clean penetration into 20-25 gauge metal framing

- Sharp point starts without a pilot hole for continuous, uninterrupted driving

- Bugle head seats flush for a paint-ready finish

- Phosphated finish rated to 48 Hour Salt Spray

## Frequently Asked Questions

What does "collated" mean on the DRP6114S?

Collated means the screws are pre-loaded into a plastic strip at a fixed spacing, so they feed automatically into a compatible autofeed screw gun attachment instead of being hand-loaded one at a time.

Will the DRP6114S work in a standard drywall screw gun?

No. Collated screws require an autofeed attachment built for strip-collated fasteners. For a standard single-screw gun, use the loose DRP6114 instead.

What size drywall screw is in each DRP6114S strip?

Each strip uses the Pro+Grip #6 x 1-1/4" fine thread screw, the same size and spec as our standard loose-pack version, just collated for speed.

# Pro+Grip #6 x 2" Collated Fine Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-2-collated-fine-thread-drywall-screw-drp6200s

SEO Title Tag: Pro+Grip #6 x 2" Collated Fine Thread Drywall Screw | Pro+Grip DRP6200S

Meta Description: The Pro+Grip DRP6200S takes our #6 x 2" fine thread drywall screw and loads it into a collated strip, so installers running autofeed screw guns can drive...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip DRP6200S takes our #6 x 2" fine thread drywall screw and loads it into a collated strip, so installers running autofeed screw guns can drive fasteners faster with less fatigue on large drywall-to-metal jobs. Same 15 TPI thread and sharp point as our standard fine thread line, just packaged for speed.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP6200S | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Collated Drywall Screw | Gauge / Diameter | #6 |
| Length | 2" | Thread Type | Fine |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.18-2.33 | Major Diameter (mm) | G2: 3.40-3.55Standard: 3.45-3.58 |
| Max Drilling Thickness | 20-25 gauge | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- Strip-collated for instant loading into autofeed screw gun attachments

- 15 TPI fine thread engineered for clean penetration into 20-25 gauge metal framing

- Sharp point starts without a pilot hole for continuous, uninterrupted driving

- Bugle head seats flush for a paint-ready finish

- Phosphated finish rated to 48 Hour Salt Spray

## Frequently Asked Questions

What does "collated" mean on the DRP6200S?

Collated means the screws are pre-loaded into a plastic strip at a fixed spacing, so they feed automatically into a compatible autofeed screw gun attachment instead of being hand-loaded one at a time.

Will the DRP6200S work in a standard drywall screw gun?

No. Collated screws require an autofeed attachment built for strip-collated fasteners. For a standard single-screw gun, use the loose DRP6200 instead.

What size drywall screw is in each DRP6200S strip?

Each strip uses the Pro+Grip #6 x 2" fine thread screw, the same size and spec as our standard loose-pack version, just collated for speed.

## Sharp Point - Fine Thread  /  Laminated

# Pro+Grip #10 x 1-1/2" Laminated Drywall Screw

Suggested URL Slug: /products/progrip-10-x-1-1-2-laminated-drywall-screw-drp10112

SEO Title Tag: Pro+Grip #10 x 1-1/2" Laminated Drywall Screw | Pro+Grip DRP10112

Meta Description: The Pro+Grip DRP10112 is a #10 x 1-1/2" laminating screw purpose-built for joining multiple layers of drywall or gypsum board together, commonly used in...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip DRP10112 is a #10 x 1-1/2" laminating screw purpose-built for joining multiple layers of drywall or gypsum board together, commonly used in shaft wall and multi-ply wall assemblies where layers need to be mechanically fastened to one another rather than to framing.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRP10112 | Application | Drywall to Drywall |
| --- | --- | --- | --- |
| Screw Type | Laminating Screw | Gauge / Diameter | #10 |
| Length | 1-1/2" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated / Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 5000 | Weight per Box (lbs) | 17.6 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.50-8.90 | Recess Depth (mm) | 2.87-3.30 |
| Minor Diameter (mm) | 3.20-3.50 | Major Diameter (mm) | 5.00-5.30 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #10 shank with 9 TPI coarse thread built to draw layered board tight

- Sharp point starts fast without a pilot hole between drywall layers

- Bugle head seats flush for smooth, paint-ready finishing

- Phosphated / Zinc finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is a laminating screw used for?

The Pro+Grip DRP10112 is used to mechanically join multiple layers of drywall or gypsum board together, a common step in shaft wall and multi-ply wall assemblies.

Is the DRP10112 used to fasten drywall to framing?

No. Laminating screws are designed to join drywall layers to each other, not to attach board to metal or wood framing. Use a standard Pro+Grip drywall screw for that step.

How many DRP10112 screws come in a box?

Each box of DRP10112 contains 5000 screws and weighs approximately 17.6 lbs, with box dimensions of 28*26*15.5.

# Sharp Point - Coarse Thread

## Sharp Point - Coarse Thread  /  Drywall Screws

# Pro+Grip #6 x 1" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-coarse-thread-drywall-screw-drpc6100

SEO Title Tag: Pro+Grip #6 x 1" Coarse Thread Drywall Screw | Pro+Grip DRPC6100

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6100 is a #6 x 1" coarse thread screw that bites hard into wood fiber for a secure hold.

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6100 is a #6 x 1" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC6100 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | 14 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.16-2.36 | Major Diameter (mm) | 3.65-3.90 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC6100 drywall screw used for?

The Pro+Grip DRPC6100 is a #6 x 1" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC6100 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC6100 screws come in a box?

Each box of DRPC6100 contains 10000 screws and weighs approximately 14 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #6 x 1-1/8" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-8-coarse-thread-drywall-screw-drpc6118

SEO Title Tag: Pro+Grip #6 x 1-1/8" Coarse Thread Drywall Screw | Pro+Grip DRPC6118

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6118 is a #6 x 1-1/8" coarse thread screw that bites hard into wood fiber for a secure...

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6118 is a #6 x 1-1/8" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC6118 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/8" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | 15.22 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.16-2.36 | Major Diameter (mm) | 3.65-3.90 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC6118 drywall screw used for?

The Pro+Grip DRPC6118 is a #6 x 1-1/8" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC6118 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC6118 screws come in a box?

Each box of DRPC6118 contains 10000 screws and weighs approximately 15.22 lbs, with box dimensions of 28*26*15.5.

# Pro+Grip #6 x 1-1/4" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-4-coarse-thread-drywall-screw-drpc6114

SEO Title Tag: Pro+Grip #6 x 1-1/4" Coarse Thread Drywall Screw | Pro+Grip DRPC6114

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6114 is a #6 x 1-1/4" coarse thread screw that bites hard into wood fiber for a secure...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6114 is a #6 x 1-1/4" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC6114 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/4" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 8000 | Weight per Box (lbs) | 13.5 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.16-2.36 | Major Diameter (mm) | 3.65-3.90 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC6114 drywall screw used for?

The Pro+Grip DRPC6114 is a #6 x 1-1/4" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC6114 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC6114 screws come in a box?

Each box of DRPC6114 contains 8000 screws and weighs approximately 13.5 lbs, with box dimensions of 28*26*15.5.

# Pro+Grip #6 x 1-5/8" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-5-8-coarse-thread-drywall-screw-drpc6158

SEO Title Tag: Pro+Grip #6 x 1-5/8" Coarse Thread Drywall Screw | Pro+Grip DRPC6158

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6158 is a #6 x 1-5/8" coarse thread screw that bites hard into wood fiber for a secure...

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6158 is a #6 x 1-5/8" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC6158 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-5/8" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 5000 | Weight per Box (lbs) | 10.65 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.16-2.36 | Major Diameter (mm) | 3.65-3.90 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC6158 drywall screw used for?

The Pro+Grip DRPC6158 is a #6 x 1-5/8" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC6158 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC6158 screws come in a box?

Each box of DRPC6158 contains 5000 screws and weighs approximately 10.65 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #6 x 2" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-2-coarse-thread-drywall-screw-drpc6200

SEO Title Tag: Pro+Grip #6 x 2" Coarse Thread Drywall Screw | Pro+Grip DRPC6200

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6200 is a #6 x 2" coarse thread screw that bites hard into wood fiber for a secure hold.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC6200 is a #6 x 2" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC6200 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #6 |
| Length | 2" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 3500 | Weight per Box (lbs) | 9.11 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.16-2.36 | Major Diameter (mm) | 3.65-3.90 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #6 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC6200 drywall screw used for?

The Pro+Grip DRPC6200 is a #6 x 2" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC6200 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC6200 screws come in a box?

Each box of DRPC6200 contains 3500 screws and weighs approximately 9.11 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #7 x 2" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-7-x-2-coarse-thread-drywall-screw-drpc7200

SEO Title Tag: Pro+Grip #7 x 2" Coarse Thread Drywall Screw | Pro+Grip DRPC7200

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC7200 is a #7 x 2" coarse thread screw that bites hard into wood fiber for a secure hold.

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC7200 is a #7 x 2" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC7200 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #7 |
| Length | 2" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 3500 | Weight per Box (lbs) | 10.49 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.40-2.70 | Major Diameter (mm) | 4.05-4.30 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #7 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC7200 drywall screw used for?

The Pro+Grip DRPC7200 is a #7 x 2" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC7200 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC7200 screws come in a box?

Each box of DRPC7200 contains 3500 screws and weighs approximately 10.49 lbs, with box dimensions of 28*26*15.5.

# Pro+Grip #8 x 2-1/2" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-8-x-2-1-2-coarse-thread-drywall-screw-drpc8212

SEO Title Tag: Pro+Grip #8 x 2-1/2" Coarse Thread Drywall Screw | Pro+Grip DRPC8212

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC8212 is a #8 x 2-1/2" coarse thread screw that bites hard into wood fiber for a secure...

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC8212 is a #8 x 2-1/2" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC8212 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #8 |
| Length | 2-1/2" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 2500 | Weight per Box (lbs) | 10.67 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.50-2.80 | Major Diameter (mm) | 4.30-4.60 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #8 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC8212 drywall screw used for?

The Pro+Grip DRPC8212 is a #8 x 2-1/2" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC8212 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC8212 screws come in a box?

Each box of DRPC8212 contains 2500 screws and weighs approximately 10.67 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip #8 x 3" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-8-x-3-coarse-thread-drywall-screw-drpc8300

SEO Title Tag: Pro+Grip #8 x 3" Coarse Thread Drywall Screw | Pro+Grip DRPC8300

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC8300 is a #8 x 3" coarse thread screw that bites hard into wood fiber for a secure hold.

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC8300 is a #8 x 3" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC8300 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #8 |
| Length | 3" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 2000 | Weight per Box (lbs) | 10.11 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.50-2.80 | Major Diameter (mm) | 4.30-4.60 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #8 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC8300 drywall screw used for?

The Pro+Grip DRPC8300 is a #8 x 3" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC8300 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC8300 screws come in a box?

Each box of DRPC8300 contains 2000 screws and weighs approximately 10.11 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip #10 x 3-1/2" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-10-x-3-1-2-coarse-thread-drywall-screw-dprc10312

SEO Title Tag: Pro+Grip #10 x 3-1/2" Coarse Thread Drywall Screw | Pro+Grip DPRC10312

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DPRC10312 is a #10 x 3-1/2" coarse thread screw that bites hard into wood fiber for a...

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DPRC10312 is a #10 x 3-1/2" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DPRC10312 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #10 |
| Length | 3-1/2" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 1000 | Weight per Box (lbs) | 8.37 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.50-8.90 | Recess Depth (mm) | 2.87-3.30 |
| Minor Diameter (mm) | 3.20-3.50 | Major Diameter (mm) | 5.00-5.30 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #10 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DPRC10312 drywall screw used for?

The Pro+Grip DPRC10312 is a #10 x 3-1/2" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DPRC10312 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DPRC10312 screws come in a box?

Each box of DPRC10312 contains 1000 screws and weighs approximately 8.37 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip #10 x 4" Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-10-x-4-coarse-thread-drywall-screw-drpc10400

SEO Title Tag: Pro+Grip #10 x 4" Coarse Thread Drywall Screw | Pro+Grip DRPC10400

Meta Description: Built for hanging drywall directly to wood framing, the Pro+Grip DRPC10400 is a #10 x 4" coarse thread screw that bites hard into wood fiber for a secure hold.

## Product Overview

Built for hanging drywall directly to wood framing, the Pro+Grip DRPC10400 is a #10 x 4" coarse thread screw that bites hard into wood fiber for a secure hold. The aggressive 9 TPI thread pulls the board tight without the wobble a fine-thread screw would have in solid wood.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC10400 | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Drywall Screw | Gauge / Diameter | #10 |
| Length | 4" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 1000 | Weight per Box (lbs) | 9.31 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.50-8.90 | Recess Depth (mm) | 2.87-3.30 |
| Minor Diameter (mm) | 3.20-3.50 | Major Diameter (mm) | 5.00-5.30 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- #10 shank with 9 TPI coarse thread built to grip wood fiber, not sheet metal

- Sharp point drives fast into standard dimensional lumber without a pilot hole

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Phosphated finish rated to 48 Hour Salt Spray

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What is the DRPC10400 drywall screw used for?

The Pro+Grip DRPC10400 is a #10 x 4" coarse thread drywall screw built for fastening drywall directly to wood studs and framing.

Can I use this screw on metal studs?

Coarse thread screws like the DRPC10400 are built to grip wood fiber. For metal stud framing, use a Pro+Grip fine thread drywall screw instead, which is cut to bite into sheet metal without stripping.

How many DRPC10400 screws come in a box?

Each box of DRPC10400 contains 1000 screws and weighs approximately 9.31 lbs, with box dimensions of 28*26*16.5.

## Sharp Point - Coarse Thread  /  Collated Screws

# Pro+Grip #6 x 1-1/4" Collated Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-4-collated-coarse-thread-drywall-screw-drpc6114s

SEO Title Tag: Pro+Grip #6 x 1-1/4" Collated Coarse Thread Drywall Screw | Pro+Grip DRPC6114S

Meta Description: The Pro+Grip DRPC6114S packages our #6 x 1-1/4" coarse thread drywall screw into a collated strip for autofeed screw guns, letting wood-framing crews hang...

## Product Overview

The Pro+Grip DRPC6114S packages our #6 x 1-1/4" coarse thread drywall screw into a collated strip for autofeed screw guns, letting wood-framing crews hang board faster without hand-loading every fastener. Same aggressive 9 TPI wood-grip thread as our standard coarse thread line.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC6114S | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Collated Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/4" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.16-2.36 | Major Diameter (mm) | 3.65-3.90 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- Strip-collated for instant loading into autofeed screw gun attachments

- 9 TPI coarse thread built to bite into wood framing, not sheet metal

- Sharp point drives fast into dimensional lumber for continuous, uninterrupted driving

- Bugle head seats flush for a paint-ready finish

- Phosphated finish rated to 48 Hour Salt Spray

## Frequently Asked Questions

What does "collated" mean on the DRPC6114S?

Collated means the screws are pre-loaded into a plastic strip at fixed spacing, so they feed automatically into a compatible autofeed screw gun attachment instead of being hand-loaded one at a time.

Will the DRPC6114S work in a standard drywall screw gun?

No. Collated screws require an autofeed attachment built for strip-collated fasteners. For a standard single-screw gun, use the loose DRPC6114 instead.

What size drywall screw is in each DRPC6114S strip?

Each strip uses the Pro+Grip #6 x 1-1/4" coarse thread screw, the same size and spec as our standard loose-pack version, built for wood framing.

# Pro+Grip #6 x 2" Collated Coarse Thread Drywall Screw

Suggested URL Slug: /products/progrip-6-x-2-collated-coarse-thread-drywall-screw-drpc6200s

SEO Title Tag: Pro+Grip #6 x 2" Collated Coarse Thread Drywall Screw | Pro+Grip DRPC6200S

Meta Description: The Pro+Grip DRPC6200S packages our #6 x 2" coarse thread drywall screw into a collated strip for autofeed screw guns, letting wood-framing crews hang board...

## Product Overview

The Pro+Grip DRPC6200S packages our #6 x 2" coarse thread drywall screw into a collated strip for autofeed screw guns, letting wood-framing crews hang board faster without hand-loading every fastener. Same aggressive 9 TPI wood-grip thread as our standard coarse thread line.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | DRPC6200S | Application | Drywall to Wood |
| --- | --- | --- | --- |
| Screw Type | Collated Drywall Screw | Gauge / Diameter | #6 |
| Length | 2" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.16-2.36 | Major Diameter (mm) | 3.65-3.90 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 48 Hour Salt Spray |

## Features & Benefits

- Strip-collated for instant loading into autofeed screw gun attachments

- 9 TPI coarse thread built to bite into wood framing, not sheet metal

- Sharp point drives fast into dimensional lumber for continuous, uninterrupted driving

- Bugle head seats flush for a paint-ready finish

- Phosphated finish rated to 48 Hour Salt Spray

## Frequently Asked Questions

What does "collated" mean on the DRPC6200S?

Collated means the screws are pre-loaded into a plastic strip at fixed spacing, so they feed automatically into a compatible autofeed screw gun attachment instead of being hand-loaded one at a time.

Will the DRPC6200S work in a standard drywall screw gun?

No. Collated screws require an autofeed attachment built for strip-collated fasteners. For a standard single-screw gun, use the loose DRPC6200 instead.

What size drywall screw is in each DRPC6200S strip?

Each strip uses the Pro+Grip #6 x 2" coarse thread screw, the same size and spec as our standard loose-pack version, built for wood framing.

# Self Drilling

## Self Drilling  /  Drywall Screws

# Pro+Grip #6 x 1-1/4" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-4-self-drilling-drywall-screw-sdz6114

SEO Title Tag: Pro+Grip #6 x 1-1/4" Self-Drilling Drywall Screw | Pro+Grip SDZ6114

Meta Description: The Pro+Grip SDZ6114 is a #6 x 1-1/4" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDZ6114 is a #6 x 1-1/4" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ6114 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/4" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 8000 | Weight per Box (lbs) | 14.9 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.40-2.60 | Major Diameter (mm) | 3.45-3.60 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 20 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ6114?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ6114 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ6114 screws come in a box?

Each box of SDZ6114 contains 8000 screws and weighs approximately 14.9 lbs.

# Pro+Grip #6 x 1-5/8" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-5-8-self-drilling-drywall-screw-sdz6158

SEO Title Tag: Pro+Grip #6 x 1-5/8" Self-Drilling Drywall Screw | Pro+Grip SDZ6158

Meta Description: The Pro+Grip SDZ6158 is a #6 x 1-5/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw...

## Product Overview

The Pro+Grip SDZ6158 is a #6 x 1-5/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ6158 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-5/8" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 5000 | Weight per Box (lbs) | 11.63 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.40-2.60 | Major Diameter (mm) | 3.45-3.60 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 20 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ6158?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ6158 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ6158 screws come in a box?

Each box of SDZ6158 contains 5000 screws and weighs approximately 11.63 lbs.

# Pro+Grip #6 x 1-7/8" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-7-8-self-drilling-drywall-screw-sdz6178

SEO Title Tag: Pro+Grip #6 x 1-7/8" Self-Drilling Drywall Screw | Pro+Grip SDZ6178

Meta Description: The Pro+Grip SDZ6178 is a #6 x 1-7/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDZ6178 is a #6 x 1-7/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ6178 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-7/8" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 3500 | Weight per Box (lbs) | 9.55 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.40-2.60 | Major Diameter (mm) | 3.45-3.60 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 20 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ6178?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ6178 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ6178 screws come in a box?

Each box of SDZ6178 contains 3500 screws and weighs approximately 9.55 lbs.

# Pro+Grip #8 x 2-3/8" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-8-x-2-3-8-self-drilling-drywall-screw-sdz8238

SEO Title Tag: Pro+Grip #8 x 2-3/8" Self-Drilling Drywall Screw | Pro+Grip SDZ8238

Meta Description: The Pro+Grip SDZ8238 is a #8 x 2-3/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDZ8238 is a #8 x 2-3/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ8238 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #8 |
| Length | 2-3/8" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 3000 | Weight per Box (lbs) | 12.69 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 18 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.85-3.05 | Major Diameter (mm) | 4.05-4.25 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 18 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ8238?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ8238 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ8238 screws come in a box?

Each box of SDZ8238 contains 3000 screws and weighs approximately 12.69 lbs.

# Pro+Grip #8 x 2-5/8" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-8-x-2-5-8-self-drilling-drywall-screw-sdz8258

SEO Title Tag: Pro+Grip #8 x 2-5/8" Self-Drilling Drywall Screw | Pro+Grip SDZ8258

Meta Description: The Pro+Grip SDZ8258 is a #8 x 2-5/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw...

## Product Overview

The Pro+Grip SDZ8258 is a #8 x 2-5/8" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ8258 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #8 |
| Length | 2-5/8" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 2000 | Weight per Box (lbs) | 9.72 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 18 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.85-3.05 | Major Diameter (mm) | 4.05-4.25 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 18 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ8258?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ8258 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ8258 screws come in a box?

Each box of SDZ8258 contains 2000 screws and weighs approximately 9.72 lbs.

# Pro+Grip #8 x 3" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-8-x-3-self-drilling-drywall-screw-sdz8300

SEO Title Tag: Pro+Grip #8 x 3" Self-Drilling Drywall Screw | Pro+Grip SDZ8300

Meta Description: The Pro+Grip SDZ8300 is a #8 x 3" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDZ8300 is a #8 x 3" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ8300 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #8 |
| Length | 3" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 2000 | Weight per Box (lbs) | 10.44 |
| Box Dimensions | 28*26*15.5 | Threads per Inch (TPI) | 18 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.85-3.05 | Major Diameter (mm) | 4.05-4.25 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 18 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ8300?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ8300 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ8300 screws come in a box?

Each box of SDZ8300 contains 2000 screws and weighs approximately 10.44 lbs.

# Pro+Grip #10 x 3-1/2" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-10-x-3-1-2-self-drilling-drywall-screw-sdz10312

SEO Title Tag: Pro+Grip #10 x 3-1/2" Self-Drilling Drywall Screw | Pro+Grip SDZ10312

Meta Description: The Pro+Grip SDZ10312 is a #10 x 3-1/2" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread...

## Product Overview

The Pro+Grip SDZ10312 is a #10 x 3-1/2" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ10312 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #10 |
| Length | 3-1/2" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 1000 | Weight per Box (lbs) | 8.51 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 8.50-8.90 | Recess Depth (mm) | 2.87-3.30 |
| Minor Diameter (mm) | 3.25-3.45 | Major Diameter (mm) | 4.62-4.82 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 16 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ10312?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ10312 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ10312 screws come in a box?

Each box of SDZ10312 contains 1000 screws and weighs approximately 8.51 lbs.

# Pro+Grip #10 x 4" Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-10-x-4-self-drilling-drywall-screw-sdz10400

SEO Title Tag: Pro+Grip #10 x 4" Self-Drilling Drywall Screw | Pro+Grip SDZ10400

Meta Description: The Pro+Grip SDZ10400 is a #10 x 4" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw...

## Product Overview

The Pro+Grip SDZ10400 is a #10 x 4" self-drilling drywall screw built for heavier 18 gauge or thicker metal framing where a sharp-point fine thread screw would struggle to start. The drill-point tip cuts its own pilot hole, so installers can drive straight through without pre-drilling.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ10400 | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Drywall Screw | Gauge / Diameter | #10 |
| Length | 4" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 1000 | Weight per Box (lbs) | 9.69 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 8.50-8.90 | Recess Depth (mm) | 2.87-3.30 |
| Minor Diameter (mm) | 3.25-3.45 | Major Diameter (mm) | 4.62-4.82 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 16 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for smooth, paint-ready drywall finishing

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drywall guns and drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ10400?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without a separate drilling step.

When should I use a self-drilling screw instead of a sharp-point screw?

Self-drilling screws like the SDZ10400 are built for heavier-gauge metal framing (18 gauge or thicker) where a standard sharp-point screw would struggle to start or would deform the tip before penetrating.

How many SDZ10400 screws come in a box?

Each box of SDZ10400 contains 1000 screws and weighs approximately 9.69 lbs.

## Self Drilling  /  Framing Screws

# Pro+Grip #6 x 7/16" Self-Drilling Framing Screw

Suggested URL Slug: /products/progrip-6-x-7-16-self-drilling-framing-screw-sdz6716

SEO Title Tag: Pro+Grip #6 x 7/16" Self-Drilling Framing Screw | Pro+Grip SDZ6716

Meta Description: The Pro+Grip SDZ6716 is a #6 x 7/16" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDZ6716 is a #6 x 7/16" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing. Its drill-point tip cuts through heavier gauge steel that a sharp-point screw cannot start on its own.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ6716 | Application | Metal to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Framing Screw | Gauge / Diameter | #6 |
| Length | 7/16" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Pan Framer Head |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 15000 | Weight per Box (lbs) | 18.05 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 7.40-8.10 | Recess Depth (mm) | 2.49-2.87 |
| Minor Diameter (mm) | 2.44-2.56 | Major Diameter (mm) | 3.35-3.53 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker steel, no pre-drilling required

- 20 TPI thread for a clean, secure metal-to-metal connection

- Pan Framer Head for a secure, low-profile fastening surface

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What does self-drilling mean on the SDZ6716?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker steel framing without a separate drilling step.

What is the SDZ6716 used for?

The Pro+Grip SDZ6716 is used for metal-to-metal framing connections, such as track to stud or stud to header, in light and heavier gauge steel framing.

How many SDZ6716 screws come in a box?

Each box of SDZ6716 contains 15000 screws and weighs approximately 18.05 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #8 x 1/2" Self-Drilling Framing Screw

Suggested URL Slug: /products/progrip-8-x-1-2-self-drilling-framing-screw-sdwz812

SEO Title Tag: Pro+Grip #8 x 1/2" Self-Drilling Framing Screw | Pro+Grip SDWZ812

Meta Description: The Pro+Grip SDWZ812 is a #8 x 1/2" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDWZ812 is a #8 x 1/2" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing. Its drill-point tip cuts through heavier gauge steel that a sharp-point screw cannot start on its own.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDWZ812 | Application | Metal to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Framing Screw | Gauge / Diameter | #8 |
| Length | 1/2" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Wafer Head - Modified Truss |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | 17.95 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 18 |
| Nominal Head Diameter (mm) | 11.10-11.60 | Recess Depth (mm) | 2.00-2.40 |
| Minor Diameter (mm) | 2.75-2.95 | Major Diameter (mm) | 4.05-4.25 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker steel, no pre-drilling required

- 18 TPI thread for a clean, secure metal-to-metal connection

- Wafer Head - Modified Truss for a secure, low-profile fastening surface

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What does self-drilling mean on the SDWZ812?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker steel framing without a separate drilling step.

What is the SDWZ812 used for?

The Pro+Grip SDWZ812 is used for metal-to-metal framing connections, such as track to stud or stud to header, in light and heavier gauge steel framing.

How many SDWZ812 screws come in a box?

Each box of SDWZ812 contains 10000 screws and weighs approximately 17.95 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #10 x 3/4" Self-Drilling Framing Screw

Suggested URL Slug: /products/progrip-10-x-3-4-self-drilling-framing-screw-sdwz1034

SEO Title Tag: Pro+Grip #10 x 3/4" Self-Drilling Framing Screw | Pro+Grip SDWZ1034

Meta Description: The Pro+Grip SDWZ1034 is a #10 x 3/4" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing.

## Product Overview

The Pro+Grip SDWZ1034 is a #10 x 3/4" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing. Its drill-point tip cuts through heavier gauge steel that a sharp-point screw cannot start on its own.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDWZ1034 | Application | Metal to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Framing Screw | Gauge / Diameter | #10 |
| Length | 3/4" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Wafer Head - Modified Truss |
| Finish | Zinc | Drive Type | #2 PHILLIPS |
| Units per Box | 5000 | Weight per Box (lbs) | 13 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 11.10-11.60 | Recess Depth (mm) | 2.00-2.40 |
| Minor Diameter (mm) | 3.25-3.45 | Major Diameter (mm) | 4.62-4.82 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker steel, no pre-drilling required

- 16 TPI thread for a clean, secure metal-to-metal connection

- Wafer Head - Modified Truss for a secure, low-profile fastening surface

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What does self-drilling mean on the SDWZ1034?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker steel framing without a separate drilling step.

What is the SDWZ1034 used for?

The Pro+Grip SDWZ1034 is used for metal-to-metal framing connections, such as track to stud or stud to header, in light and heavier gauge steel framing.

How many SDWZ1034 screws come in a box?

Each box of SDWZ1034 contains 5000 screws and weighs approximately 13 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #10 x 3/4" Self-Drilling Framing Screw

Suggested URL Slug: /products/progrip-10-x-3-4-self-drilling-framing-screw-sdhz1034

SEO Title Tag: Pro+Grip #10 x 3/4" Self-Drilling Framing Screw | Pro+Grip SDHZ1034

Meta Description: The Pro+Grip SDHZ1034 is a #10 x 3/4" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing.

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDHZ1034 is a #10 x 3/4" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing. Its drill-point tip cuts through heavier gauge steel that a sharp-point screw cannot start on its own.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDHZ1034 | Application | Metal to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Framing Screw | Gauge / Diameter | #10 |
| Length | 3/4" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Hex Washer Head |
| Finish | Zinc | Drive Type | 5/16" HEX |
| Units per Box | 6000 | Weight per Box (lbs) | 16.4 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 9.75-10.50 | Recess Depth (mm) | N/A |
| Minor Diameter (mm) | 3.25-3.45 | Major Diameter (mm) | 4.62-4.82 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker steel, no pre-drilling required

- 16 TPI thread for a clean, secure metal-to-metal connection

- Hex Washer Head for a secure, low-profile fastening surface

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- 5/16" HEX drive for consistent engagement with standard drivers

## Frequently Asked Questions

What does self-drilling mean on the SDHZ1034?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker steel framing without a separate drilling step.

What is the SDHZ1034 used for?

The Pro+Grip SDHZ1034 is used for metal-to-metal framing connections, such as track to stud or stud to header, in light and heavier gauge steel framing.

How many SDHZ1034 screws come in a box?

Each box of SDHZ1034 contains 6000 screws and weighs approximately 16.4 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #10 x 1-1/4" Self-Drilling Framing Screw

Suggested URL Slug: /products/progrip-10-x-1-1-4-self-drilling-framing-screw-sdhz10114

SEO Title Tag: Pro+Grip #10 x 1-1/4" Self-Drilling Framing Screw | Pro+Grip SDHZ10114

Meta Description: The Pro+Grip SDHZ10114 is a #10 x 1-1/4" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing.

## Product Overview

The Pro+Grip SDHZ10114 is a #10 x 1-1/4" self-drilling framing screw built for metal-to-metal connections in 18 gauge or thicker steel framing. Its drill-point tip cuts through heavier gauge steel that a sharp-point screw cannot start on its own.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDHZ10114 | Application | Metal to Metal |
| --- | --- | --- | --- |
| Screw Type | Self-Drilling Framing Screw | Gauge / Diameter | #10 |
| Length | 1-1/4" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Hex Washer Head |
| Finish | Zinc | Drive Type | 5/16" HEX |
| Units per Box | 4000 | Weight per Box (lbs) | 15.22 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 9.75-10.50 | Recess Depth (mm) | N/A |
| Minor Diameter (mm) | 3.25-3.45 | Major Diameter (mm) | 4.62-4.82 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Zinc-Plated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker steel, no pre-drilling required

- 16 TPI thread for a clean, secure metal-to-metal connection

- Hex Washer Head for a secure, low-profile fastening surface

- Zinc finish for Zinc-Plated, Standard Interior Corrosion Resistance

- 5/16" HEX drive for consistent engagement with standard drivers

## Frequently Asked Questions

What does self-drilling mean on the SDHZ10114?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker steel framing without a separate drilling step.

What is the SDHZ10114 used for?

The Pro+Grip SDHZ10114 is used for metal-to-metal framing connections, such as track to stud or stud to header, in light and heavier gauge steel framing.

How many SDHZ10114 screws come in a box?

Each box of SDHZ10114 contains 4000 screws and weighs approximately 15.22 lbs, with box dimensions of 28*26*16.5.

## Self Drilling  /  Collated Screws

# Pro+Grip #6 x 1-1/4" Collated Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-1-4-collated-self-drilling-drywall-screw-sdz6114s

SEO Title Tag: Pro+Grip #6 x 1-1/4" Collated Self-Drilling Drywall Screw | Pro+Grip SDZ6114S

Meta Description: The Pro+Grip SDZ6114S loads our #6 x 1-1/4" self-drilling drywall screw into a collated strip, giving installers working in 18 gauge or thicker metal...

Popular Item: Yes, flag as a best-seller on the page

## Product Overview

The Pro+Grip SDZ6114S loads our #6 x 1-1/4" self-drilling drywall screw into a collated strip, giving installers working in 18 gauge or thicker metal framing the speed of an autofeed screw gun without giving up the drill-point tip needed to start in heavier steel.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ6114S | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Collated Self-Drilling Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-1/4" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.40-2.60 | Major Diameter (mm) | 3.45-3.60 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Phosphated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Strip-collated for instant loading into autofeed screw gun attachments

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 20 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for a paint-ready finish

- Phosphated finish for Phosphated, Standard Interior Corrosion Resistance

## Frequently Asked Questions

What does "collated" mean on the SDZ6114S?

Collated means the screws are pre-loaded into a plastic strip at fixed spacing, so they feed automatically into a compatible autofeed screw gun attachment instead of being hand-loaded one at a time.

What does self-drilling mean on this screw?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without pre-drilling.

Will the SDZ6114S work in a standard drywall screw gun?

No. Collated screws require an autofeed attachment built for strip-collated fasteners.

# Pro+Grip #6 x 1-7/8" Collated Self-Drilling Drywall Screw

Suggested URL Slug: /products/progrip-6-x-1-7-8-collated-self-drilling-drywall-screw-sdz6178s

SEO Title Tag: Pro+Grip #6 x 1-7/8" Collated Self-Drilling Drywall Screw | Pro+Grip SDZ6178S

Meta Description: The Pro+Grip SDZ6178S loads our #6 x 1-7/8" self-drilling drywall screw into a collated strip, giving installers working in 18 gauge or thicker metal...

## Product Overview

The Pro+Grip SDZ6178S loads our #6 x 1-7/8" self-drilling drywall screw into a collated strip, giving installers working in 18 gauge or thicker metal framing the speed of an autofeed screw gun without giving up the drill-point tip needed to start in heavier steel.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | SDZ6178S | Application | Drywall to Metal |
| --- | --- | --- | --- |
| Screw Type | Collated Self-Drilling Drywall Screw | Gauge / Diameter | #6 |
| Length | 1-7/8" | Thread Type | Fine |
| Point Type | Self-Drilling Point | Head Type | Bugle Head |
| Finish | Phosphated | Drive Type | #2 PHILLIPS |
| Units per Box | 10000 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 20 |
| Nominal Head Diameter (mm) | 8.00-8.30 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.40-2.60 | Major Diameter (mm) | 3.45-3.60 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | Phosphated, Standard Interior Corrosion Resistance |

## Features & Benefits

- Strip-collated for instant loading into autofeed screw gun attachments

- Self-drilling point cuts its own pilot hole in 18 gauge or thicker metal, no pre-drilling required

- 20 TPI thread for a clean, secure hold in heavier-gauge steel framing

- Bugle head seats flush for a paint-ready finish

- Phosphated finish for Phosphated, Standard Interior Corrosion Resistance

## Frequently Asked Questions

What does "collated" mean on the SDZ6178S?

Collated means the screws are pre-loaded into a plastic strip at fixed spacing, so they feed automatically into a compatible autofeed screw gun attachment instead of being hand-loaded one at a time.

What does self-drilling mean on this screw?

A self-drilling screw has a drill-bit-style tip that cuts its own pilot hole as it drives, letting installers fasten straight into 18 gauge or thicker metal framing without pre-drilling.

Will the SDZ6178S work in a standard drywall screw gun?

No. Collated screws require an autofeed attachment built for strip-collated fasteners.

# Concrete and Cement Board Screws

# Pro+Grip #8 x 1-1/4" Sharp Point Cement Board Screw

Suggested URL Slug: /products/progrip-8-x-1-1-4-sharp-point-cement-board-screw-cps8114

SEO Title Tag: Pro+Grip #8 x 1-1/4" Sharp Point Cement Board Screw | Pro+Grip CPS8114

Meta Description: The Pro+Grip CPS8114 is a #8 x 1-1/4" cement board screw built for fastening cement board to wood framing.

## Product Overview

The Pro+Grip CPS8114 is a #8 x 1-1/4" cement board screw built for fastening cement board to wood framing. Its ribbed Flat Head with Ribs countersinks cleanly into the cement board surface, and the Gray Ruspert Coating (1500 HR Salt Spray Rated) finish is rated for 1500 Hour Salt Spray (Ruspert Coating), well beyond what standard interior fasteners see.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | CPS8114 | Application | Cement Board to Wood |
| --- | --- | --- | --- |
| Screw Type | Cement Board Screw | Gauge / Diameter | #8 |
| Length | 1-1/4" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Flat Head with Ribs |
| Finish | Gray Ruspert Coating (1500 HR Salt Spray Rated) | Drive Type | #2 PHILLIPS |
| Units per Box | 5000 | Weight per Box (lbs) | 12.1 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 10.00-10.50 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.50-2.80 | Major Diameter (mm) | 4.30-4.60 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1500 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- #8 shank with 9 TPI coarse thread built for cement board installations

- Sharp point drives fast into wood framing without a pilot hole

- Ribbed Flat Head with Ribs countersinks flush into cement board for a clean finish

- Gray Ruspert Coating (1500 HR Salt Spray Rated) finish rated to 1500 Hour Salt Spray (Ruspert Coating)

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the CPS8114 cement board screw used for?

The Pro+Grip CPS8114 is used to fasten cement board underlayment or backer board to framing, commonly ahead of tile installation in wet areas.

Does the CPS8114 need a pilot hole?

No, the sharp point drives directly into wood framing without pre-drilling.

How many CPS8114 screws come in a box?

Each box of CPS8114 contains 5000 screws and weighs approximately 12.1 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #8 x 1-5/8" Sharp Point Cement Board Screw

Suggested URL Slug: /products/progrip-8-x-1-5-8-sharp-point-cement-board-screw-cps8158

SEO Title Tag: Pro+Grip #8 x 1-5/8" Sharp Point Cement Board Screw | Pro+Grip CPS8158

Meta Description: The Pro+Grip CPS8158 is a #8 x 1-5/8" cement board screw built for fastening cement board to wood framing.

## Product Overview

The Pro+Grip CPS8158 is a #8 x 1-5/8" cement board screw built for fastening cement board to wood framing. Its ribbed Flat Head with Ribs countersinks cleanly into the cement board surface, and the Gray Ruspert Coating (1500 HR Salt Spray Rated) finish is rated for 1500 Hour Salt Spray (Ruspert Coating), well beyond what standard interior fasteners see.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | CPS8158 | Application | Cement Board to Wood |
| --- | --- | --- | --- |
| Screw Type | Cement Board Screw | Gauge / Diameter | #8 |
| Length | 1-5/8" | Thread Type | Coarse |
| Point Type | Sharp Point | Head Type | Flat Head with Ribs |
| Finish | Gray Ruspert Coating (1500 HR Salt Spray Rated) | Drive Type | #2 PHILLIPS |
| Units per Box | 4000 | Weight per Box (lbs) | 12.04 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 9 |
| Nominal Head Diameter (mm) | 10.00-10.50 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.50-2.80 | Major Diameter (mm) | 4.30-4.60 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1500 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- #8 shank with 9 TPI coarse thread built for cement board installations

- Sharp point drives fast into wood framing without a pilot hole

- Ribbed Flat Head with Ribs countersinks flush into cement board for a clean finish

- Gray Ruspert Coating (1500 HR Salt Spray Rated) finish rated to 1500 Hour Salt Spray (Ruspert Coating)

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the CPS8158 cement board screw used for?

The Pro+Grip CPS8158 is used to fasten cement board underlayment or backer board to framing, commonly ahead of tile installation in wet areas.

Does the CPS8158 need a pilot hole?

No, the sharp point drives directly into wood framing without pre-drilling.

How many CPS8158 screws come in a box?

Each box of CPS8158 contains 4000 screws and weighs approximately 12.04 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip #8 x 1-1/4" Self-Drilling Cement Board Screw

Suggested URL Slug: /products/progrip-8-x-1-1-4-self-drilling-cement-board-screw-cpsd8114

SEO Title Tag: Pro+Grip #8 x 1-1/4" Self-Drilling Cement Board Screw | Pro+Grip CPSD8114

Meta Description: The Pro+Grip CPSD8114 is a #8 x 1-1/4" cement board screw built for fastening cement board to metal framing.

## Product Overview

The Pro+Grip CPSD8114 is a #8 x 1-1/4" cement board screw built for fastening cement board to metal framing. Its ribbed Flat Head with Ribs countersinks cleanly into the cement board surface, and the Gray Ruspert Coating (1500 HR Salt Spray Rated) finish is rated for 1500 Hour Salt Spray (Ruspert Coating), well beyond what standard interior fasteners see.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | CPSD8114 | Application | Cement Board to Metal |
| --- | --- | --- | --- |
| Screw Type | Cement Board Screw | Gauge / Diameter | #8 |
| Length | 1-1/4" | Thread Type | Coarse |
| Point Type | Self-Drilling Point | Head Type | Flat Head with Ribs |
| Finish | Gray Ruspert Coating (1500 HR Salt Spray Rated) | Drive Type | #2 PHILLIPS |
| Units per Box | 5000 | Weight per Box (lbs) | 12.85 |
| Box Dimensions | 28*26*13.5 | Threads per Inch (TPI) | 18 |
| Nominal Head Diameter (mm) | 10.00-10.50 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.85-3.05 | Major Diameter (mm) | 4.05-4.25 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | 1500 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- #8 shank with 18 TPI coarse thread built for cement board installations

- Self-drilling point cuts through 18 gauge or thicker metal framing without pre-drilling

- Ribbed Flat Head with Ribs countersinks flush into cement board for a clean finish

- Gray Ruspert Coating (1500 HR Salt Spray Rated) finish rated to 1500 Hour Salt Spray (Ruspert Coating)

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the CPSD8114 cement board screw used for?

The Pro+Grip CPSD8114 is used to fasten cement board underlayment or backer board to framing, commonly ahead of tile installation in wet areas.

Does the CPSD8114 need a pilot hole?

No. The self-drilling point cuts through 18 gauge or thicker metal framing on its own.

How many CPSD8114 screws come in a box?

Each box of CPSD8114 contains 5000 screws and weighs approximately 12.85 lbs, with box dimensions of 28*26*13.5.

# Pro+Grip #8 x 1-5/8" Self-Drilling Cement Board Screw

Suggested URL Slug: /products/progrip-8-x-1-5-8-self-drilling-cement-board-screw-cpsd8158

SEO Title Tag: Pro+Grip #8 x 1-5/8" Self-Drilling Cement Board Screw | Pro+Grip CPSD8158

Meta Description: The Pro+Grip CPSD8158 is a #8 x 1-5/8" cement board screw built for fastening cement board to metal framing.

## Product Overview

The Pro+Grip CPSD8158 is a #8 x 1-5/8" cement board screw built for fastening cement board to metal framing. Its ribbed Flat Head with Ribs countersinks cleanly into the cement board surface, and the Gray Ruspert Coating (1500 HR Salt Spray Rated) finish is rated for 1500 Hour Salt Spray (Ruspert Coating), well beyond what standard interior fasteners see.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | CPSD8158 | Application | Cement Board to Metal |
| --- | --- | --- | --- |
| Screw Type | Cement Board Screw | Gauge / Diameter | #8 |
| Length | 1-5/8" | Thread Type | Coarse |
| Point Type | Self-Drilling Point | Head Type | Flat Head with Ribs |
| Finish | Gray Ruspert Coating (1500 HR Salt Spray Rated) | Drive Type | #2 PHILLIPS |
| Units per Box | 4000 | Weight per Box (lbs) | 12.84 |
| Box Dimensions | 28*26*16.5 | Threads per Inch (TPI) | 18 |
| Nominal Head Diameter (mm) | 10.00-10.50 | Recess Depth (mm) | 2.64-3.00 |
| Minor Diameter (mm) | 2.85-3.05 | Major Diameter (mm) | 4.05-4.25 |
| Max Drilling Thickness | 18 gauge or thicker | Corrosion Resistance | 1500 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- #8 shank with 18 TPI coarse thread built for cement board installations

- Self-drilling point cuts through 18 gauge or thicker metal framing without pre-drilling

- Ribbed Flat Head with Ribs countersinks flush into cement board for a clean finish

- Gray Ruspert Coating (1500 HR Salt Spray Rated) finish rated to 1500 Hour Salt Spray (Ruspert Coating)

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the CPSD8158 cement board screw used for?

The Pro+Grip CPSD8158 is used to fasten cement board underlayment or backer board to framing, commonly ahead of tile installation in wet areas.

Does the CPSD8158 need a pilot hole?

No. The self-drilling point cuts through 18 gauge or thicker metal framing on its own.

How many CPSD8158 screws come in a box?

Each box of CPSD8158 contains 4000 screws and weighs approximately 12.84 lbs, with box dimensions of 28*26*16.5.

# Pro+Grip 1/4" x 1-1/4" Concrete Screw

Suggested URL Slug: /products/progrip-1-4-x-1-1-4-concrete-screw-mhdp14114

SEO Title Tag: Pro+Grip 1/4" x 1-1/4" Concrete Screw | Pro+Grip MHDP14114

Meta Description: The Pro+Grip MHDP14114 is a 1/4" x 1-1/4" concrete screw built for anchoring metal components directly into concrete or masonry.

## Product Overview

The Pro+Grip MHDP14114 is a 1/4" x 1-1/4" concrete screw built for anchoring metal components directly into concrete or masonry. The high-low thread pattern cuts its own internal threads into pre-drilled concrete, eliminating the need for a separate anchor.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | MHDP14114 | Application | Metal to Concrete |
| --- | --- | --- | --- |
| Screw Type | Concrete Screw | Gauge / Diameter | 1/4" |
| Length | 1-1/4" | Thread Type | High-Low |
| Point Type | Sharp Point | Head Type | Hex Washer Head with Phil/Slot |
| Finish | Blue Ruspert Coating (1000 HR Salt Spray Rated) | Drive Type | 5/16" HEX |
| Units per Box | 100 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 10.10-11.00 | Recess Depth (mm) | To Be Confirmed |
| Minor Diameter (mm) | 5.15-5.35 | Major Diameter (mm) | 6.20-6.50 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1000 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- 1/4" diameter, 1-1/4" high-low thread concrete screw for direct-to-concrete anchoring

- Sharp point and aggressive thread form cut internal threads into pre-drilled concrete or masonry

- Hex Washer Head with Phil/Slot for a secure, flush-mounted connection

- Blue Ruspert Coating (1000 HR Salt Spray Rated) finish rated to 1000 Hour Salt Spray (Ruspert Coating)

- 5/16" HEX drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the MHDP14114 concrete screw used for?

The Pro+Grip MHDP14114 is used to anchor metal brackets, track, and fixtures directly into concrete or masonry, without a separate expansion anchor.

Do I need to pre-drill before installing this concrete screw?

Yes. Concrete screws require a pre-drilled pilot hole sized to the screw diameter; the high-low thread then cuts its own internal threads as it's driven in.

How many MHDP14114 screws come in a box?

Each box of MHDP14114 contains 100 screws.

# Pro+Grip 1/4" x 1-3/4" Concrete Screw

Suggested URL Slug: /products/progrip-1-4-x-1-3-4-concrete-screw-mhdp14134

SEO Title Tag: Pro+Grip 1/4" x 1-3/4" Concrete Screw | Pro+Grip MHDP14134

Meta Description: The Pro+Grip MHDP14134 is a 1/4" x 1-3/4" concrete screw built for anchoring metal components directly into concrete or masonry.

## Product Overview

The Pro+Grip MHDP14134 is a 1/4" x 1-3/4" concrete screw built for anchoring metal components directly into concrete or masonry. The high-low thread pattern cuts its own internal threads into pre-drilled concrete, eliminating the need for a separate anchor.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | MHDP14134 | Application | Metal to Concrete |
| --- | --- | --- | --- |
| Screw Type | Concrete Screw | Gauge / Diameter | 1/4" |
| Length | 1-3/4" | Thread Type | High-Low |
| Point Type | Sharp Point | Head Type | Hex Washer Head with Phil/Slot |
| Finish | Blue Ruspert Coating (1000 HR Salt Spray Rated) | Drive Type | 5/16" HEX |
| Units per Box | 100 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 15 |
| Nominal Head Diameter (mm) | 10.10-11.00 | Recess Depth (mm) | To Be Confirmed |
| Minor Diameter (mm) | 5.15-5.35 | Major Diameter (mm) | 6.20-6.50 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1000 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- 1/4" diameter, 1-3/4" high-low thread concrete screw for direct-to-concrete anchoring

- Sharp point and aggressive thread form cut internal threads into pre-drilled concrete or masonry

- Hex Washer Head with Phil/Slot for a secure, flush-mounted connection

- Blue Ruspert Coating (1000 HR Salt Spray Rated) finish rated to 1000 Hour Salt Spray (Ruspert Coating)

- 5/16" HEX drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the MHDP14134 concrete screw used for?

The Pro+Grip MHDP14134 is used to anchor metal brackets, track, and fixtures directly into concrete or masonry, without a separate expansion anchor.

Do I need to pre-drill before installing this concrete screw?

Yes. Concrete screws require a pre-drilled pilot hole sized to the screw diameter; the high-low thread then cuts its own internal threads as it's driven in.

How many MHDP14134 screws come in a box?

Each box of MHDP14134 contains 100 screws.

# Pro+Grip 3/16" x 1-1/4" Concrete Screw

Suggested URL Slug: /products/progrip-3-16-x-1-1-4-concrete-screw-mhdp316114

SEO Title Tag: Pro+Grip 3/16" x 1-1/4" Concrete Screw | Pro+Grip MHDP316114

Meta Description: The Pro+Grip MHDP316114 is a 3/16" x 1-1/4" concrete screw built for anchoring metal components directly into concrete or masonry.

## Product Overview

The Pro+Grip MHDP316114 is a 3/16" x 1-1/4" concrete screw built for anchoring metal components directly into concrete or masonry. The high-low thread pattern cuts its own internal threads into pre-drilled concrete, eliminating the need for a separate anchor.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | MHDP316114 | Application | Metal to Concrete |
| --- | --- | --- | --- |
| Screw Type | Concrete Screw | Gauge / Diameter | 3/16" |
| Length | 1-1/4" | Thread Type | High-Low |
| Point Type | Sharp Point | Head Type | Hex Washer Head with Phil/Slot |
| Finish | Blue Ruspert Coating (1000 HR Salt Spray Rated) | Drive Type | 5/16" HEX |
| Units per Box | 100 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 9.50-10.00 | Recess Depth (mm) | To Be Confirmed |
| Minor Diameter (mm) | 4.10-4.30 | Major Diameter (mm) | 5.00-5.25 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1000 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- 3/16" diameter, 1-1/4" high-low thread concrete screw for direct-to-concrete anchoring

- Sharp point and aggressive thread form cut internal threads into pre-drilled concrete or masonry

- Hex Washer Head with Phil/Slot for a secure, flush-mounted connection

- Blue Ruspert Coating (1000 HR Salt Spray Rated) finish rated to 1000 Hour Salt Spray (Ruspert Coating)

- 5/16" HEX drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the MHDP316114 concrete screw used for?

The Pro+Grip MHDP316114 is used to anchor metal brackets, track, and fixtures directly into concrete or masonry, without a separate expansion anchor.

Do I need to pre-drill before installing this concrete screw?

Yes. Concrete screws require a pre-drilled pilot hole sized to the screw diameter; the high-low thread then cuts its own internal threads as it's driven in.

How many MHDP316114 screws come in a box?

Each box of MHDP316114 contains 100 screws.

# Pro+Grip 3/16" x 1-3/4" Concrete Screw

Suggested URL Slug: /products/progrip-3-16-x-1-3-4-concrete-screw-mhdp316134

SEO Title Tag: Pro+Grip 3/16" x 1-3/4" Concrete Screw | Pro+Grip MHDP316134

Meta Description: The Pro+Grip MHDP316134 is a 3/16" x 1-3/4" concrete screw built for anchoring metal components directly into concrete or masonry.

## Product Overview

The Pro+Grip MHDP316134 is a 3/16" x 1-3/4" concrete screw built for anchoring metal components directly into concrete or masonry. The high-low thread pattern cuts its own internal threads into pre-drilled concrete, eliminating the need for a separate anchor.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | MHDP316134 | Application | Metal to Concrete |
| --- | --- | --- | --- |
| Screw Type | Concrete Screw | Gauge / Diameter | 3/16" |
| Length | 1-3/4" | Thread Type | High-Low |
| Point Type | Sharp Point | Head Type | Hex Washer Head with Phil/Slot |
| Finish | Blue Ruspert Coating (1000 HR Salt Spray Rated) | Drive Type | 5/16" HEX |
| Units per Box | 100 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 9.50-10.00 | Recess Depth (mm) | To Be Confirmed |
| Minor Diameter (mm) | 4.10-4.30 | Major Diameter (mm) | 5.00-5.25 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1000 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- 3/16" diameter, 1-3/4" high-low thread concrete screw for direct-to-concrete anchoring

- Sharp point and aggressive thread form cut internal threads into pre-drilled concrete or masonry

- Hex Washer Head with Phil/Slot for a secure, flush-mounted connection

- Blue Ruspert Coating (1000 HR Salt Spray Rated) finish rated to 1000 Hour Salt Spray (Ruspert Coating)

- 5/16" HEX drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the MHDP316134 concrete screw used for?

The Pro+Grip MHDP316134 is used to anchor metal brackets, track, and fixtures directly into concrete or masonry, without a separate expansion anchor.

Do I need to pre-drill before installing this concrete screw?

Yes. Concrete screws require a pre-drilled pilot hole sized to the screw diameter; the high-low thread then cuts its own internal threads as it's driven in.

How many MHDP316134 screws come in a box?

Each box of MHDP316134 contains 100 screws.

# Pro+Grip 3/16" x 1-1/4" Concrete Screw

Suggested URL Slug: /products/progrip-3-16-x-1-1-4-concrete-screw-mdp316114

SEO Title Tag: Pro+Grip 3/16" x 1-1/4" Concrete Screw | Pro+Grip MDP316114

Meta Description: The Pro+Grip MDP316114 is a 3/16" x 1-1/4" concrete screw built for anchoring metal components directly into concrete or masonry.

## Product Overview

The Pro+Grip MDP316114 is a 3/16" x 1-1/4" concrete screw built for anchoring metal components directly into concrete or masonry. The high-low thread pattern cuts its own internal threads into pre-drilled concrete, eliminating the need for a separate anchor.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | MDP316114 | Application | Metal to Concrete |
| --- | --- | --- | --- |
| Screw Type | Concrete Screw | Gauge / Diameter | 3/16" |
| Length | 1-1/4" | Thread Type | High-Low |
| Point Type | Sharp Point | Head Type | Flat Head |
| Finish | Blue Ruspert Coating (1000 HR Salt Spray Rated) | Drive Type | #2 PHILLIPS |
| Units per Box | 100 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 9.00-10.00 | Recess Depth (mm) | 2.64-3.15 |
| Minor Diameter (mm) | 4.10-4.30 | Major Diameter (mm) | 5.00-5.35 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1000 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- 3/16" diameter, 1-1/4" high-low thread concrete screw for direct-to-concrete anchoring

- Sharp point and aggressive thread form cut internal threads into pre-drilled concrete or masonry

- Flat Head for a secure, flush-mounted connection

- Blue Ruspert Coating (1000 HR Salt Spray Rated) finish rated to 1000 Hour Salt Spray (Ruspert Coating)

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the MDP316114 concrete screw used for?

The Pro+Grip MDP316114 is used to anchor metal brackets, track, and fixtures directly into concrete or masonry, without a separate expansion anchor.

Do I need to pre-drill before installing this concrete screw?

Yes. Concrete screws require a pre-drilled pilot hole sized to the screw diameter; the high-low thread then cuts its own internal threads as it's driven in.

How many MDP316114 screws come in a box?

Each box of MDP316114 contains 100 screws.

# Pro+Grip 3/16" x 1-3/4" Concrete Screw

Suggested URL Slug: /products/progrip-3-16-x-1-3-4-concrete-screw-mdp316134

SEO Title Tag: Pro+Grip 3/16" x 1-3/4" Concrete Screw | Pro+Grip MDP316134

Meta Description: The Pro+Grip MDP316134 is a 3/16" x 1-3/4" concrete screw built for anchoring metal components directly into concrete or masonry.

## Product Overview

The Pro+Grip MDP316134 is a 3/16" x 1-3/4" concrete screw built for anchoring metal components directly into concrete or masonry. The high-low thread pattern cuts its own internal threads into pre-drilled concrete, eliminating the need for a separate anchor.

Every Pro+Grip fastener is built to a tighter tolerance and backed by verified corrosion testing, so installers and distributors get a screw that performs the same box after box.

## Key Specifications

| Stock Code | MDP316134 | Application | Metal to Concrete |
| --- | --- | --- | --- |
| Screw Type | Concrete Screw | Gauge / Diameter | 3/16" |
| Length | 1-3/4" | Thread Type | High-Low |
| Point Type | Sharp Point | Head Type | Flat Head |
| Finish | Blue Ruspert Coating (1000 HR Salt Spray Rated) | Drive Type | #2 PHILLIPS |
| Units per Box | 100 | Weight per Box (lbs) | N/A |
| Box Dimensions | N/A | Threads per Inch (TPI) | 16 |
| Nominal Head Diameter (mm) | 9.00-10.00 | Recess Depth (mm) | 2.64-3.15 |
| Minor Diameter (mm) | 4.10-4.30 | Major Diameter (mm) | 5.00-5.35 |
| Max Drilling Thickness | N/A | Corrosion Resistance | 1000 Hour Salt Spray (Ruspert Coating) |

## Features & Benefits

- 3/16" diameter, 1-3/4" high-low thread concrete screw for direct-to-concrete anchoring

- Sharp point and aggressive thread form cut internal threads into pre-drilled concrete or masonry

- Flat Head for a secure, flush-mounted connection

- Blue Ruspert Coating (1000 HR Salt Spray Rated) finish rated to 1000 Hour Salt Spray (Ruspert Coating)

- #2 PHILLIPS drive for consistent engagement with standard drivers

## Frequently Asked Questions

What is the MDP316134 concrete screw used for?

The Pro+Grip MDP316134 is used to anchor metal brackets, track, and fixtures directly into concrete or masonry, without a separate expansion anchor.

Do I need to pre-drill before installing this concrete screw?

Yes. Concrete screws require a pre-drilled pilot hole sized to the screw diameter; the high-low thread then cuts its own internal threads as it's driven in.

How many MDP316134 screws come in a box?

Each box of MDP316134 contains 100 screws.